HOWARD COUNTY SMOKE ALARM CODE REQUIREMENTS

General Requirements

- All battery powered smoke alarms shall be sealed, tamper resistant units incorporating a silence/hush button and using long-life (10 year) batteries. All smoke alarms shall be UL Listed.

- All smoke alarms, battery powered or electric powered, shall be replaced when they are more than 10 years old (from date of manufacture).

- When electric (AC) powered smoke alarms require replacement they shall be replaced with an electric powered smoke alarm with a battery back-up, never with a battery powered smoke alarm.

- A smoke detector installed as part of an approved fire alarm system is an acceptable alternative to an electric (AC) powered smoke alarm.

- Each sleeping room occupied by a deaf or hard-of-hearing person shall be equipped with a smoke alarm suitable to alert a deaf or hard-of-hearing person.

Installation Requirements:

Homeowners are required to meet the smoke alarm code requirements based upon when their home was constructed (permit issued) as per the following chart:

| Residential One, Two, and Three Family Homes Including Townhouses |  |  |
| --- | --- | --- |
| If your home was constructed |  | The smoke alarm requirements are as follows : |
| Prior to 7/1/1982 |  | Shall have a minimum of one (1) smoke alarm installed: On every level of the home, including basements and occupied attics. Outside each sleeping area. Smoke alarms may be either battery or hard wired directly to the building’s power supply. |
| Between 7/1/1982 | 1/1/1989 | Shall have a minimum of one (1) smoke alarm installed: On every level of the home, including basements and occupied attics. Outside each sleeping area. The smoke alarm outside the sleeping area must be electric (AC) powered. |
| Between 1/1/1989 | 7/1/1990 | Shall have a minimum of one electric (AC) powered smoke alarm installed: On every level of the home, including basements and occupied attics. Outside each sleeping area. The smoke alarms shall be interconnected to alarm simultaneously. |
| Between 7/1/1990 | 10/10/2001 | Same as 1/1/89 to 7/1/1990 requirements plus: The smoke alarms shall have integral battery backup. |
| After 10/10/2001 |  | Same as 1/1/89 to 10/10/2001 requirements plus: Smoke alarms shall be long lasting (10 year) with battery backup. A smoke alarm shall be installed in each sleeping room |
| Residential Multi-Family (Apartments, Condos) |  |  |
| Prior 1/1/1989 |  | Multi‐family residential buildings shall have smoke alarms hardwired to the buildings power supply. |
| 1/1/1989 |  | All requirements same as stated in dates above for Single family Residential. |

Note: Home businesses or other situations, such as family home daycare, assisted living facilities and rental properties have additional smoke alarm code requirements.

Definitions:

Sleeping Area – a space that includes one or more sleeping rooms and a hall or common area immediately adjacent to any sleeping room

Sleeping Room – an enclosed room with a bed or similar piece of furniture arranged to be used as a bedroom

Smoke Alarm – a single or multiple station device that detects visible or invisible products of combustion and includes a built-in internal alarm signal

Smoke Detector – a system-connected smoke sensing device tied to a fire alarm control panel or a household fire warning panel

UL Listed – tested by a nationally recognized testing laboratory to comply with Underwriters Laboratories standard for single or multiple station smoke alarms